Table 2.1  The IAS Instruction Set

| Instruction Type | Opcode | Symbolic Representation | Description |
| --- | --- | --- | --- |
| Data transfer | 00001010 | LOAD MQ | Transfer contents of register MQ to the accumulator AC |
|  | 00001001 | LOAD MQ,M(X) | Transfer contents of memory location X to MQ |
|  | 00100001 | STOR M(X) | Transfer contents of accumulator to memory location X |
|  | 00000001 | LOAD M(X) | Transfer M(X) to the accumulator |
|  | 00000010 | LOAD –M(X) | Transfer –M(X) to the accumulator |
|  | 00000011 | LOAD ¦M(X)¦ | Transfer absolute value of M(X) to the accumulator |
|  | 00000100 | LOAD –¦M(X)¦ | Transfer –¦M(X)¦ to the accumulator |
| Unconditional branch | 00001101 | JUMP M(X,0:19) | Take next instruction from left half of M(X) |
|  | 00001110 | JUMP M(X,20:39) | Take next instruction from right half of M(X) |
| Conditional branch | 00001111 | JUMP+ M(X,0:19) | If number in the accumulator is nonnegative, take next instruction from left half of M(X) |
|  | 00010000 | JUMP+ M(X,20:39) | If number in the accumulator is nonnegative, take next instruction from right half of M(X) |
| Arithmetic | 00000101 | ADD M(X) | Add M(X) to AC; put the result in AC |
|  | 00000111 | ADD ¦M(X)¦ | Add ¦M(X)¦ to AC; put the result in AC |
|  | 00000110 | SUB M(X) | Subtract M(X) from AC; put the result in AC |
|  | 00001000 | SUB ¦M(X)¦ | Subtract ¦M(X)¦ from AC; put the remainder in AC |
|  | 00001011 | MUL M(X) | Multiply M(X) by MQ; put most significant bits of result in AC, put least significant bits in MQ |
|  | 00001100 | DIV M(X) | Divide AC by M(X); put the quotient in MQ and the remainder in AC |
|  | 00010100 | LSH | Multiply accumulator by 2; i.e., shift left one bit position |
|  | 00010101 | RSH | Divide accumulator by 2; i.e., shift right one position |
| Address modify | 00010010 | STOR M(X,8:19) | Replace left address field at M(X) by 12 rightmost bits of AC |
|  | 00010011 | STOR M(X,28:39) | Replace right address field at M(X) by 12 rightmost bits of AC |

Table 2.2  Computer Generations

| Generation | Approximate Dates | Technology | Typical Speed (operations per second) |
| --- | --- | --- | --- |
| 1 | 1946–1957 | Vacuum tube | 40,000 |
| 2 | 1958–1964 | Transistor | 200,000 |
| 3 | 1965–1971 | Small and medium scale integration | 1,000,000 |
| 4 | 1972–1977 | Large scale integration | 10,000,000 |
| 5 | 1978–1991 | Very large scale integration | 100,000,000 |
| 6 | 1991- | Ultra large scale integration | 1,000,000,000 |

# Table 2.4  Key Characteristics of the System/360 Family

| Characteristic | Model 30 | Model 40 | Model 50 | Model 65 | Model 75 |
| --- | --- | --- | --- | --- | --- |
| Maximum memory size (bytes) | 64K | 256K | 256K | 512K | 512K |
| Data rate from memory (Mbytes/sec) | 0.5 | 0.8 | 2.0 | 8.0 | 16.0 |
| Processor cycle time s) | 1.0 | 0.625 | 0.5 | 0.25 | 0.2 |
| Relative speed | 1 | 3.5 | 10 | 21 | 50 |
| Maximum number of data channels | 3 | 3 | 4 | 6 | 6 |
| Maximum data rate on one channel (Kbytes/s) | 250 | 400 | 800 | 1250 | 1250 |

Table 2.7 Examples of Embedded Systems and Their Markets

| Market | Embedded Device |
| --- | --- |
| Automotive | Ignition system Engine control Brake system |
| Consumer electronics | Digital and analog televisions Set-top boxes (DVDs, VCRs, Cable boxes) Personal digital assistants (PDAs) Kitchen appliances (refrigerators, toasters, microwave ovens) Automobiles Toys/games Telephones/cell phones/pagers Cameras Global positioning systems |
| Industrial control | Robotics and controls systems for manufacturing Sensors |
| Medical | Infusion pumps Dialysis machines Prosthetic devices Cardiac monitors |
| Office automation | Fax machine Photocopier Printers Monitors Scanners |

Table 2.8 ARM Evolution

| Family | Notable Features | Cache | Typical MIPS @ MHz |
| --- | --- | --- | --- |
| ARM1 | 32-bit RISC | None |  |
| ARM2 | Multiply and swap instructions; Integrated memory management unit, graphics and I/O processor | None | 7 MIPS @ 12 MHz |
| ARM3 | First use of processor cache | 4 KB unified | 12 MIPS @ 25 MHz |
| ARM6 | First to support 32-bit addresses; floating-point unit | 4 KB unified | 28 MIPS @ 33 MHz |
| ARM7 | Integrated SoC | 8 KB unified | 60 MIPS @ 60 MHz |
| ARM8 | 5-stage pipeline; static branch prediction | 8 KB unified | 84 MIPS @ 72 MHz |
| ARM9 |  | 16 KB/16 KB | 300 MIPS @ 300 MHz |
| ARM9E | Enhanced DSP instructions | 16 KB/16 KB | 220 MIPS @ 200 MHz |
| ARM10E | 6-stage pipeline | 32 KB/32 KB |  |
| ARM11 | 9-stage pipeline | Variable | 740 MIPS @ 665 MHz |
| Cortex | 13-stage superscalar pipeline | Variable | 2000 MIPS @ 1 GHz |
| XScale | Applications processor; 7-stage pipeline | 32 KB/32 KB L1 512 KB L2 | 1000 MIPS @ 1.25 GHz |

DSP = digital signal processor

SoC = system on a chip

Table 2.9  Performance Factors and System Attributes

|  | Ic | p | m | k |  |
| --- | --- | --- | --- | --- | --- |
| Instruction set architecture | X | X |  |  |  |
| Compiler technology | X | X | X |  |  |
| Processor implementation |  | X |  |  | X |
| Cache and memory hierarchy |  |  |  | X | X |